Appendix 1

- Basing on the conservation of momentum theory, the pre-impact speed of the Suzuki can be determined by applying the equation: momentum pre-impact equals to momentum post impact (for angle collision):

=    ----- [1]

Where:

– Pre-impact speed of the Suzuki

– Weight of Suzuki = 1220kg (Suzuki 1100kg (manufacturer specification) + 70kg (male driver) + 50kg (female passenger)

– Post impact speed of Suzuki = 33.9kmph (see Appendix 2)

– Departure angle of Suzuki after impact = 100 (approximately derived based on the accident scene photographs in the KAR Report).

– Weight of Honda = 1445kg (Honda 1370kg (manufacturer specification) + 70kg (male driver) + 5kg (2-year-old son)

– Post impact speed of Honda = 26.34kmph (see Appendix 2)

– Departure angle of Honda after impact = 700 (approximately derived based on the accident scene photographs in the KAR Report)

– Pre-impact speed of Honda = 29.68kmph (see Appendix 3)

– Approach angle of Honda before impact = 900 (right angle intersection)

The pre-impact speed of the Suzuki is calculated to be:

= 1220 x 33.9 x cos100+1445 x 26.34 x cos700-(1445 x 29.68 x cos900)1220

= 1220 x 33.9 x 0.984+1445 x 26.34 x 0.342-(1445 x 29.68 x 0)1220

= 40696.272 +13016.964-01220

= 53713.2361220

Pre-impact speed of Suzuki = 44kmph

Reference:

[1] 	Technical Accident Investigation & Reconstruction Course, Society of Automotive Engineers, Australasia. Chapter 15, Dynamics and Momentum

Appendix 2

- In order to apply the conservation of momentum equation, the post impact speed of the Suzuki will have to be established first. By applying equations of liner motion:

Post impact speed =   ----- [1]

Where:

– Post impact speed of the Suzuki

– Post impact distance travelled by the Suzuki = 6.46mm (basing on my road measurements from the point of impact to its final rest location).

– Acceleration rate () where  = 9.81m/s; = 0.7 (recommended 0.7 for asphalt road in dry weather condition) ----- [2] = -6.867 (negative as the Suzuki was decelerating)

– Velocity end = 0m/s (the Suzuki had come to a complete stop)

Therefore, the post impact speed of Suzuki was:

= 02-(2 x-6.867 x 6.46)

=88.721

= 9.419m/s

Post impact speed of Suzuki = 33.9kmph (9.419 x 3.6)

- In order to apply the conservation of momentum equation, the post impact speed of the Honda will also have to be established first. By applying equations of liner motion:

Post impact speed =   ----- [1]

Where:

– Post impact speed of the Honda

– Post impact distance travelled by the Honda = 3.9mm (basing on my road measurements from the point of impact to its final rest location).

– Acceleration rate () where  = 9.81m/s; = 0.7 (recommended 0.7 for asphalt road in dry weather condition) ----- [2] = -6.867 (negative as the Honda was decelerating)

– Velocity end = 0m/s (the Honda had come to a complete stop)

Therefore, the post impact speed of Honda was:

= 02-(2 x-6.867 x 3.9)

= 53.562

= 7.318m/s

Post impact speed of Honda = 26.34kmph (7.318 x 3.6).

Reference:

[1]	Technical Accident Investigation & Reconstruction Course, Society of Automotive Engineers, Australasia. Chapter 15, Dynamics, Momentum;

[2] S.H Backaitis, Reconstruction of Motor Vehicle Accident: A Technical Compendium, Society of Automotive Engineers, Friction Application in Accident Reconstruction;

Appendix 3

- Having determined the post impact speed of the Suzuki and the Honda in Appendix 2, we can now determine the pre-impact speed of the Honda by applying the equation:

=    ----- [1]

Where:

– Pre-impact speed of the Honda

– Weight of Suzuki = 1220kg (Suzuki 1100kg (manufacturer specification) + 70kg (male driver) + 50kg (female passenger)

– Post impact speed of Suzuki = 33.9kmph (see Appendix 2)

– Departure angle of Suzuki after impact = 100 (approximately derived based on the accident scene photographs in the KAR Report).

– Weight of Honda = 1445kg (Honda 1370kg (manufacturer specification) + 70kg (male driver) + 5kg (2-year-old boy)

– Post impact speed of Suzuki = 26.34kmph (see Appendix 2)

– Departure angle of Honda after impact = 700 (approximately derived based on the accident scene photographs in the KAR Report)

– Approach angle of Honda before impact = 900 (right angle intersection)

The pre-impact speed of the Honda is calculated to be:

= 1220 x 33.9 x sin100+(1445 x 26.34 x sin700)1445 xsin902

= 1220 x 33.9 x 0.173+(1445 x 26.34 x 0.939)1445 x 1

= 7154.934+35739.5601445

= 42894.4941445

Pre-impact speed of Honda = 29.68kmph

Reference:

[1] 	Technical Accident Investigation & Reconstruction Course, Society of Automotive Engineers, Australasia. Chapter 15, Dynamics and Momentum